### BAMBU LABS X1:

### Step 1: Download Bambu Studio

- Open your web browser and go to the Bambu Lab Studio official website: Bambu Lab Studio.

### Step 2: Launch Bambu Studio

- Once the installation is complete, click the Finish button.

- Find the Bambu Studio icon on your desktop or in your applications list.

- Double-click to open Bambu Studio.

### Step 3: Connect Your 3D Printer

- In Bambu Studio, go to the Device icon in the top menu bar.

- Add your Bambu Labs X1 3D Printer (NAME: mrsd-4) by clicking Add Printer.

  - If you’re using Wi-Fi, ensure the printer is connected to the same network as your computer. – Use (cmu-device wifi)

  - Enter the printer’s IP address or search for it automatically.

### Step 4: Configure Print Settings

- After connecting the printer, choose the material you're going to print with.

  - Select PLA, ABS, PETG, or any other material compatible with the printer.

  - Currently the printer is using PLA material.

- Adjust your print settings for layer height, infill, and support options as needed.

### Step 5: Slice the 3D Model

- Open your 3D model file (usually in .stl or .obj format) by clicking File → Open or drag and drop it into the Bambu Studio window. Your cad file should be on the Prepare page.

- Check the orientation and scale of your model to ensure it’s ready for printing. Use the Auto-orient button on the top to orient it appropriately for 3D printing.

- Click the Slice plate button to prepare the file for printing.

- Import multiple components to be print at once by clicking File → Import

### Step 6: Start Printing

- Once your model is sliced, click the Print Plate button.

- You can choose to print via USB or over Wi-Fi if the printer and your computer are on the same network.

- Monitor your print progress in Bambu Studio, and ensure everything is set up correctly.

DEBUG:

If you are unable to connect to the 3D printer directly using your laptop:

Step 1: Install Bambu labs app on your mobile device.

Step 2: Create an account and login to the app.

Step 3: Scan the barcode given in the bottom right corner of the printer.

Step 4: Your user account will now be connected to the 3D printer

Step 5: Login using the same account on your desktop and print the component.

Step 6: Please logout your user account from the printer after use.

### PRUSA 3D printer:

### Step 1: Install a Slicer Software

Students can choose and install any slicer software like PrusaSlicer, Ultimaker Cura, or Simplify3D. Here’s a quick installation guide for PrusaSlicer:

#### Installing PrusaSlicer

- Download PrusaSlicer from the official Prusa website: PrusaSlicer Download.

### Step 2: Prepare the 3D Model

- Open your slicer software and load your 3D model file (in .stl or .obj format).

- Configure your slicer settings based on the material you’re using:

  - PLA, PETG, etc.

  - Choose the appropriate layer height, infill, supports, and print speed.

- Make sure the model is properly scaled and positioned for printing.

### Step 3: Slice the Model

- Once your 3D model is ready, click the Slice button in your slicer software.

  - This will generate a .gcode file which is the machine-readable format for 3D printers.

- Save the .gcode file to your computer.

### Step 4: Prepare the USB Drive

- Take the USB drive provided for the Prusa 3D Printer.

- Plug the USB into your computer.

- Open the USB drive and look for a folder named MRSD_2026.

  - If the folder doesn’t exist, create a new folder with that name.

- Copy your sliced .gcode file into the MRSD_2026 folder on the USB drive.

### Step 5: Insert USB into the Prusa 3D Printer

- Safely eject the USB from your computer and insert it into the Prusa 3D Printer's USB slot.

### Step 6: Select Your File on the Printer

- Turn the Prusa 3D Printer on.

- Use the knob on the printer’s front panel to navigate through the menu.

- Navigate to Print from SD/USB or Select File

- Find the MRSD_2026 folder and locate your .gcode file.

- Select your file and press the knob to start the print.

### Step 7: Start the Print

- Once the print starts, monitor the initial layer to ensure proper adhesion.

- Stay nearby for the first few minutes to ensure everything is working correctly.